| NO. | Performance Parameters | TT | FF | SS | FnSp | SnFp |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | DC Gain(dB) | 71.9366 | 68.9745 | 72.8455 | 70.8103 | 72.7023 |
| 2 | 3-dB Bandwidth(Hz) | 4.2395k | 6.0042k | 3.7281k | 4.8376k | 3.8657k |
| 3 | Slew Rate(V/us) | 17.8 | 17.9 | 16.9 | 17.4 | 18 |
| 4 | Settling Time(us) | 0.68 | 0.78 | 0.62 | 0.69 | 0.53 |
| 5 | CMRR(dB) | 210.7917 | 202.7448 | 214.8733 | 206.5053 | 206.3872 |
| 6 | PSRR(VDD)(dB) | 209.5586 | 226.8923 | 209.6000 | 222.4707 | 204.4728 |
| 7 | PSRR(GND)(dB) | 221.7783 | 225.2085 | 216.3918 | 219.7673 | 235.7218 |
| 8 | Unity-gain Frequency(Hz) | 16.2195M | 16.3384M | 15.8460M | 16.2718M | 16.1411M |
| 9 | Phase Margin (°) | 74.2417 | 73.9909 | 74.5330 | 74.3321 | 74.1580 |
| 10 | Power(uW) | 467.9814 | 468.3307 | 467.4536 | 467.7367 | 468.1831 |

AIC HW5

111511076 陳彥宇

Problem 1:

Fig 1. TT Bode Plot (DC Gain, Bandwidth, Unity-gain Freq., Phase Margin)

Fig 2. TT Slew Rate & Settling Time

Fig 3. TT .measure(DC Gain, Bandwidth, Unity-gain Freq., Phase Margin)

Fig 4. TT .measure(Common-mode Gain)

Fig 5. TT .measure(Gain from VDD)

Fig 6. TT .measure(Gain from GND)

Fig 7. TT Power

Fig 8. FF Bode Plot (DC Gain, Bandwidth, Unity-gain Freq., Phase Margin)

Fig 9. FF Slew Rate & Settling Time

Fig 10. FF .measure(DC Gain, Bandwidth, Unity-gain Freq., Phase Margin)

Fig 11. FF .measure(Common-mode Gain)

Fig 12. FF .measure(Gain from VDD)

Fig 13. FF .measure(Gain from GND)

Fig 14. FF Power

Fig 15. SS Bode Plot (DC Gain, Bandwidth, Unity-gain Freq., Phase Margin)

Fig 16. SS Slew Rate & Settling Time

Fig 17. SS .measure(DC Gain, Bandwidth, Unity-gain Freq., Phase Margin)

Fig 18. SS .measure(Common-mode Gain)

Fig 19. SS .measure(Gain from VDD)

Fig 20. SS .measure(Gain from GND)

Fig 21. SS Power

Fig 22. FnSp Bode Plot (DC Gain, Bandwidth, Unity-gain Freq., Phase Margin)

Fig 23. FnSp Slew Rate & Settling Time

Fig 24. FnSp .measure(DC Gain, Bandwidth, Unity-gain Freq., Phase Margin)

Fig 25. FnSp TT .measure(Common-mode Gain)

Fig 26. FnSp TT .measure(Gain from VDD)

Fig 27. FnSp TT .measure(Gain from GND)

Fig 28. FnSp Power

Fig 29. SnFp Bode Plot (DC Gain, Bandwidth, Unity-gain Freq., Phase Margin)

Fig 30. SnFp Slew Rate & Settling Time

Fig 31. SnFp .measure(DC Gain, Bandwidth, Unity-gain Freq., Phase Margin)

Fig 32. SnFp .measure(Common-mode Gain)

Fig 33. SnFp .measure(Gain from VDD)

Fig 34. SnFp .measure(Gain from GND)

Fig 35. SnFp Power

Problem 2:

Fig 36. M2 W/L = 60.1u/4u TT Bode Plot (DC Gain, Bandwidth, Unity-gain Freq., Phase Margin)

Fig 37. M2 W/L = 60.1u/4u TT Slew Rate & Settling Time

Fig 38. M2 W/L = 60.1u/4u TT .measure(DC Gain, Bandwidth, Unity-gain Freq., Phase Margin)

Fig 39. M2 W/L = 60.1u/4u TT .measure(Common-mode Gain)

Fig 40. M2 W/L = 60.1u/4u TT .measure(Gain from VDD)

Fig 41. M2 W/L = 60.1u/4u TT .measure(Gain from GND)

Fig 42. M2 W/L = 60.1u/4u TT .measure(Power)

| NO. | Performance Parameters | TT M2 W/L = 60.1u/4u | TT M2 W/L = 60u/4u |
| --- | --- | --- | --- |
| 1 | DC Gain(dB) | 59.1438 | 71.9366 |
| 2 | 3-dB Bandwidth(Hz) | 18.2747k | 4.2395k |
| 3 | Slew Rate(V/us) | 17.8 | 17.8 |
| 4 | Settling Time(us) | 0.68 | 0.68 |
| 5 | CMRR(dB) | 67.0102 | 210.7917 |
| 6 | PSRR(VDD)(dB) | 56.8229 | 209.5586 |
| 7 | PSRR(GND)(dB) | 228.8452 | 221.7783 |
| 8 | Unity-gain Frequency(Hz) | 16.0809M | 16.2195M |
| 9 | Phase Margin (°) | 74.4547 | 74.2417 |
| 10 | Power(uW) | 468.0421 | 467.9814 |

Fig 43. M2 W/L = 60.1u/4u & M2 W/L = 60u/4u TT Bode Plot Comparison

Comparison & Discussion:

Because the widths of the MOSFETs in the differential amplifier are mismatched, it causes imbalance in the differential pair, leading to several undesirable effects:

1. Mismatch in Transconductance (gm)

The transconductance gm of a MOSFET depends on its width (W). Because the width are mismatched, the gm values of the two sides are different. This results in an unequal current flow in the differential branches, even when the input voltages are balanced. This imbalance reduces the symmetry of the circuit and affects the differential amplifier's performance.

2. Reduced Common-Mode Rejection Ratio (CMRR)

The differential amplifier is designed to amplify differential signals and reject common-mode signals. As we can see from the .tf results above, the mismatched widths degrade the common-mode rejection ratio (CMRR) because the differential pair no longer perfectly cancels common-mode signals.

3. Reduced Power Supply Rejection Ration (PSRR)

The same reasons CMRR is reduced can be applied here. Unequal gm makes the amplifier more sensitive to variations in the power supply.

4. Reduced Gain and Linearity

The differential gain of the amplifier depends on the matching of the transistors. Width mismatch has reduced the gain and affected the linearity of the amplifier. Its gain reduced by approximately 12.8dB compared to the perfectly matched amplifier.

5. Phase Response

The width of a MOSFET directly influences its parasitic capacitances. Therefore, the first pole of the mismatched amplifier is shifted to the right by approximately 15.7Hz.

6. Slew Rate & Settling Time

The slew rate and settling time seem to be unaffected by the mismatch.

Code for Problems 1 & 2:

1. ***-----------------------***

2. ***        setting        ***

3. ***-----------------------***

4. .lib "~/U18_HSPICE_Model/mm180_reg18_v124.lib" tt

5. .TEMP 25

6. .op 0.5m

7. ***-----------------------***

8. ***       simulation      ***

9. ***-----------------------***

10.

11. .option post

12. .option accurate=1

13. .ac    DEC  100 1 500Meg

14. .probe AC P(Vout+,Vout-)

15. .probe AC Vdb(Vout+,Vout-)

16. .probe AC VP(Vout+,Vout-)

17. .tran 0.1n 20u

18. .probe tran V(Vout+, Vout-)

19. .tf   V(Vout+, Vout-)   V1

20.

21. ***-----------------------***

22. ***       measure         ***

23. ***-----------------------***

24. .meas AC DCgainDB max Vdb(Vout+,Vout-)

25. .meas AC DCgainV/V max V(Vout+,Vout-)

26. .meas AC f3db when Vdb(Vout+,Vout-)='DCgainDB-3.0'

27. .meas AC funity when Vdb(Vout+,Vout-)='0'

28. .meas AC wgc_phase find VP(Vout+,Vout-) when Vdb(Vout+,Vout-)='0'

29. .meas AC PM param = 'wgc_phase + 180'

30. .meas tran avg_power avg power

31.

32. .meas tran vout_max param = '1.25'

33. .meas tran delta_t trig V(Vout+,Vout-) val='1.25*0.1' rise = 1

34. +targ V(Vout+,Vout-) val='1.25*0.9' rise = 1

35. .meas tran slew_rate param = "(1.25*0.9 - 1.25*0.1) / delta_t"

36.

37. ***-----------------------***

38. ***      power/input      ***

39. ***-----------------------***

40. Vsupply     VDD     vss       1.8V

41. V1          Vin+    Common    DC      0.8V      AC  0.5V   0

42. V2          Vin-    Common    DC      0.8V      AC  0.5V   180

43. V_common    Common    vss     DC      0V

44. Vgnd    GND 0   0V

45. VSS     vss GND 0V

46.

47. V3  Vcmo     vss    0.9V

48. I1  VDD      Vb2    50uA

49. I2  Vb3      vss    50uA

50. I3  Vb4      vss    50uA

51.

52. ***-----------------------***

53. ***        circuit        ***

54. ***-----------------------***

55. Mb1     Vb2     Vb2     vss     vss     n_18_mm w=3u   l=4u     ad=0.48u*3u as=0.48u*3u pd=0.96u+3u ps=0.96u+3u

56. Mb2     Vb3     Vb3     VDD     VDD     p_18_mm w=15u  l=4u     ad=0.48u*15u as=0.48u*15u pd=0.96u+15u ps=0.96u+15u

57. Mb3     N1      Vb4     VDD     VDD     p_18_mm w=90u  l=4u     ad=0.48u*90u as=0.48u*90u pd=0.96u+90u ps=0.96u+90u

58. Mb4     Vb4     Vb3     N1      VDD     p_18_mm w=50u  l=4u     ad=0.48u*50u as=0.48u*50u pd=0.96u+50u ps=0.96u+50u

59.

60. MSS     N2      Vb1     vss     vss     n_18_mm w=150u l=8u     ad=0.48u*150u as=0.48u*150u pd=0.96u+150u ps=0.96u+150u

61. M1      N3      Vin+    N2      vss     n_18_mm w=60u  l=4u     ad=0.48u*60u as=0.48u*60u pd=0.96u+60u ps=0.96u+60u

62. M2      N4      Vin-    N2      vss     n_18_mm w=60u  l=4u     ad=0.48u*60u as=0.48u*60u pd=0.96u+60u ps=0.96u+60u

63. M3      Vout-   Vb2     N3      vss     n_18_mm w=65u  l=4u     ad=0.48u*65u as=0.48u*65u pd=0.96u+65u ps=0.96u+65u

64. M4      Vout+   Vb2     N4      vss     n_18_mm w=65u  l=4u     ad=0.48u*65u as=0.48u*65u pd=0.96u+65u ps=0.96u+65u

65.

66. CL1     Vout-   vss     5p

67. CL2     Vout+   vss     5p

68.

69. M5      Vout-   Vb3     N5      VDD     p_18_mm w=50u  l=4u     ad=0.48u*50u as=0.48u*50u pd=0.96u+50u ps=0.96u+50u

70. M6      Vout+   Vb3     N6      VDD     p_18_mm w=50u  l=4u     ad=0.48u*50u as=0.48u*50u pd=0.96u+50u ps=0.96u+50u

71. M7      N5      Vb4     VDD     VDD     p_18_mm w=90u  l=4u     ad=0.48u*90u as=0.48u*90u pd=0.96u+90u ps=0.96u+90u

72. M8      N6      Vb4     VDD     VDD     p_18_mm w=90u  l=4u     ad=0.48u*90u as=0.48u*90u pd=0.96u+90u ps=0.96u+90u

73.

74. M9      Vb1     Vb1     vss     vss     n_18_mm w=8u   l=8u     ad=0.48u*8u as=0.48u*8u pd=0.96u+8u ps=0.96u+8u

75. M10     N7      N7      vss     vss     n_18_mm w=8u   l=8u     ad=0.48u*8u as=0.48u*8u pd=0.96u+8u ps=0.96u+8u

76. M11     N7      Vout-   N8      N8      p_18_mm w=5u   l=1u     ad=0.48u*5u as=0.48u*5u pd=0.96u+5u ps=0.96u+5u

77. M12     Vb1     Vcmo    N8      N8      p_18_mm w=5u   l=1u     ad=0.48u*5u as=0.48u*5u pd=0.96u+5u ps=0.96u+5u

78. M13     Vb1     Vcmo    N9      N9      p_18_mm w=5u   l=1u     ad=0.48u*5u as=0.48u*5u pd=0.96u+5u ps=0.96u+5u

79. M14     N7      Vout+   N9      N9      p_18_mm w=5u   l=1u     ad=0.48u*5u as=0.48u*5u pd=0.96u+5u ps=0.96u+5u

80. M15     N8      Vb4     VDD     VDD     p_18_mm w=9u   l=4u     ad=0.48u*9u as=0.48u*9u pd=0.96u+9u ps=0.96u+9u

81. M16     N9      Vb4     VDD     VDD     p_18_mm w=9u   l=4u     ad=0.48u*9u as=0.48u*9u pd=0.96u+9u ps=0.96u+9u

82.

83. *Original circuit measures DC gain, 3-dB bandwidth, power, unity gain freq., Phase Margin

84.

85. .alter

86. *measures Acm

87. .tf   V(Vout+, Vout-)   V_common

88. V1          Vin+    Common   DC      0.8V

89. V2          Vin-    Common   DC      0.8V

90. V_common    Common    vss    DC      0V     AC  0.5

91.

92. .alter

93. *measures Slew Rate, Settling Time

94. *                           PULSE(V1    V2  Tdelay  Trise   Tfall   Pwidth  Period)

95. V1          Vin+      vss   PULSE(0V  1.8V  1us     0ns     0ns 30us 50us)

96. V2          Vin-      vss    DC      0.8V

97. V_common    Common    vss    DC      0V

98.

99. .alter

100. *measures gain from GND

101. .tf   V(Vout+, Vout-)   VSS

102. V_common    Common    vss    DC      0V

103. VSS         vss       GND    DC      0V     AC 1

104.

105. .alter

106. *measures gain from VDD

107. .tf   V(Vout+, Vout-)   Vsupply

108. V_common    Common    vss    DC      0V

109. VSS         vss       GND    DC      0V

110. Vsupply     VDD       vss    DC      1.8V   AC 1

111.

112. .end